--------------------------------------------------------------------------------

--

--   AM2901 Benchmark -- Q_Reg component

--

-- Source: AMD data book

--

-- VHDL Benchmark author Indraneel Ghosh

--                       University Of California, Irvine, CA 92717

--

-- Developed on Nov 1, 1991

--

--------------------------------------------------------------------------------

The Q register operation is controlled by three most significant bits

( I8, I7, I6) in the instruction. Thus there are eight possible cases.

The test vectors in "tv_q_reg.vhdl" test the Q register component in

all modes of operation for each of the eight cases.

---------------------------------------------------------------------

(1)  Load F into Q

I <= "000000000";

F <= "0111";

Q0 <= 'Z';

Q3 <= 'Z';

---------------------------------------------------------------------

(2) Do nothing

I <= "001000000";

F <= "0000";

Q0 <= 'Z';

Q3 <= 'Z';

---------------------------------------------------------------------

(3) Do nothing

I <= "010000000";

F <= "0000";

Q0 <= 'Z';

Q3 <= 'Z';

---------------------------------------------------------------------

(4) Do nothing

I <= "011000000";

F <= "0000";

Q0 <= 'Z';

Q3 <= 'Z';

---------------------------------------------------------------------

(5) Down shift Q with input 1

I <= "100000000";

F <= "0000";

Q0 <= 'Z';

Q3 <= '1';

---------------------------------------------------------------------

(6) Down shift Q with input 0

I <= "100000000";

F <= "0000";

Q0 <= 'Z';

Q3 <= '0';

--------------------------------------------------------------------

(7) Do nothing

I <= "101000000";

F <= "0000";

Q0 <= 'Z';

Q3 <= 'Z';

---------------------------------------------------------------------

(8) Up shift Q with input 1

I <= "110000000";

F <= "0000";

Q0 <= '1';

Q3 <= 'Z';

---------------------------------------------------------------------

(9) Up shift Q with input 0

I <= "110000000";

F <= "0000";

Q0 <= '0';

Q3 <= 'Z';

---------------------------------------------------------------------

(10) Do nothing

I <= "111000000";

F <= "0000";

Q0 <= 'Z';

Q3 <= 'Z';

---------------------------------------------------------------------