--------------------------------------------------------------------------------

--

--   AM2901 Benchmark

--

-- Source: AMD data book

--

-- VHDL Benchmark author Indraneel Ghosh

--                       University Of California, Irvine, CA 92717

--

-- Developed on Jan 1, 1992

--------------------------------------------------------------------------------

The "output_and_shifter" block (RAM) operation is controlled by three most

significant bits ( I8, I7, I6) in the instruction. Thus there are eight

possible cases.

The test vectors in "tv_op_sh.vhdl" test the "output_and_shifter" component in

all modes of operation for each of the eight cases.

-------------------------------------------------------------------------

(1)  Y gets F.

F <= "0101";

A <= "1110";

Q <= "0000";

I <= "000000000";

OEBAR <= '0';

-------------------------------------------------------------------------

(2)  Y gets tri-stated.

F <= "0101";

A <= "0111";

Q <= "0000";

I <= "000000000";

OEBAR <= '1';

-------------------------------------------------------------------------

(3)  Y gets F.

F <= "1110";

A <= "0110";

Q <= "0000";

I <= "001000000";

OEBAR <= '0';

-------------------------------------------------------------------------

(4)  Y gets A.

F <= "1110";

A <= "0110";

Q <= "0000";

I <= "010000000";

OEBAR <= '0';

-------------------------------------------------------------------------

(5)  Y gets F.

F <= "1110";

A <= "0111";

Q <= "0000";

I <= "011000000";

OEBAR <= '0';

-------------------------------------------------------------------------

(6)  Y gets F. RAM0 gets F(0). Q0 gets Q(0)

F <= "1001";

A <= "1000";

Q <= "0100";

I <= "100000000";

OEBAR <= '0';

-------------------------------------------------------------------------

(7)  Y gets F. RAM0 gets F(0). Q0 gets Q(0)

F <= "0110";

A <= "1110";

Q <= "1101";

I <= "101000000";

OEBAR <= '0';

-------------------------------------------------------------------------

(8) Y gets F. RAM3 gets F(3). Q3 gets Q(3)

F <= "0100";

A <= "0010";

Q <= "1000";

I <= "110000000";

OEBAR <= '0';

-------------------------------------------------------------------------

(9) Y gets F. RAM3 gets F(3). Q3 gets Q(3)

F <= "0001";

A <= "1100";

Q <= "1000";

I <= "111000000";

OEBAR <= '0';

-------------------------------------------------------------------------