--------------------------------------------------------------------------------

--

--   AM2901 Benchmark

--

-- Source: AM2901 Data Book

--

-- author: Indraneel Ghosh

--         University Of California, Irvine, CA 92717

--

-- Written on Nov 1, 1991

--

--------------------------------------------------------------------------------

1) INTRODUCTION:

The Am 2901 four-bit microprocessor slice (from Advanced Micro Devices Inc.)

is a high-speed cascadable ALU intended for use in CPUs, peripheral

controllers and programmable microprocessors.

( NOTE: It will be helpful to refer to the block diagram of Am 2901 in the

postscript file "am2901.ps". )

The data is 4 bits wide at all points.

------------------------------------------------------------------------------

(1.1)      FUNCTIONAL BLOCKS

Its main functional blocks are as follows :

** A 16-word by four bit two port RAM, with an up/down shifter at the input.

## Port A is an output port.

## Port B is a bidirectional port.

** A register ( called Q ) with an up/down shifter at the input

** An ALU source selector which select two inputs out of the following :

## Port A of RAM

## Port B of RAM

## Q register output

## External data input

## Logical 0

** A 4-bit ALU, capable of performing arithmetic and logical functions on the

selected source words.

** A destination selector which decides :

## whether to load the ALU output ( with or without shifting) into the

RAM.

## whether to load the ALU output ( with or without shifting) or the

Q register contents ( with shifting ) into the Q register

## whether the ALU output or the Port A contents should be forwarded to

the External data output.

( NOTE: The destination selector is not shown as an explicit block in the

figure. Its parts are included in the RAM, Q-register and output

selector.)

------------------------------------------------------------------------------

(1.2)    PORTS

INPUT AND OUTPUT PORTS:

_____________________________________________________________________________

|  PORT | TYPE  |BIT  |                                                       |

|       |       |WIDTH|      DESCRIPTION                                      |

|_______|_______|_____|_______________________________________________________|

|     I |    in | 9   | Instruction word (discussed later)                    |

|_______|_______|_____|_______________________________________________________|

|  Aadd |    in | 4   | Address input to RAM ( for READ)                      |

|_______|_______|_____|_______________________________________________________|

|  Badd |    in | 4   | Address input to RAM ( for READ / WRITE )             |

|_______|_______|_____|_______________________________________________________|

|     D |    in | 4   | Data input to chip                                    |

|_______|_______|_____|_______________________________________________________|

|     Y |   out | 4   | Data output from chip                                 |

|_______|_______|_____|_______________________________________________________|

|  RAM0 | inout | 1   | Up/down shifter port connected to LSB of RAM          |

|_______|_______|_____|_______________________________________________________|

|  RAM3 | inout | 1   | Up/down shifter port connected to MSB of RAM          |

|_______|_______|_____|_______________________________________________________|

|    Q0 | inout | 1   | Up/down shifter port connected to LSB of Q-register   |

|_______|_______|_____|_______________________________________________________|

|    Q3 | inout | 1   | Up/down shifter port connected to MSB of Q-register   |

|_______|_______|_____|_______________________________________________________|

|   CLK |    in | 1   | clock                                                 |

|_______|_______|_____|_______________________________________________________|

|    C0 |    in | 1   | Carry input to ALU                                    |

|_______|_______|_____|_______________________________________________________|

| OEbar |    in | 1   | Tri-state driver input ( if this is not asserted,     |

|       |       |     |   the data output Y will be tri-stated to HIGH-Z )    |

|_______|_______|_____|_______________________________________________________|

|    C4 |   out | 1   | Carry output from ALU                                 |

|_______|_______|_____|_______________________________________________________|

|  Gbar |   out | 1   | Generate term from ALU for carry lookahead            |

|_______|_______|_____|_______________________________________________________|

|  Pbar |   out | 1   | Propagate term from ALU for carry lookahead           |

|_______|_______|_____|_______________________________________________________|

|   OVR |   out | 1   | Overflow output from ALU (this signals that an        |

|       |       |     | overflow has occurred, while performing the operation)|

|_______|_______|_____|_______________________________________________________|

|    F3 |   out | 1   | MSB of the ALU output                                 |

|_______|_______|_____|_______________________________________________________|

|   F30 |   out | 1   | Zero signal ( asserted if the all 4 bits of ALU       |

|       |       |     |      output are zero).                                |

|_______|_______|_____|_______________________________________________________|

NOTE: In the models, Aadd and Badd are represented as integers instead of

bit vectors. This is done to avoid cumbersome and unneccessary

computation, while accessing the RAM.

------------------------------------------------------------------------------

(1.3)      THE INSTRUCTION SET:

The Am 2901 has a 9-bit instruction, which has three 3-bit fields whose

functions are as follows :

** I2 downto I0 : controls ALU source selector

** I5 downto I3 : controls ALU function

** I8 downto I6 : controls destination selector

(1.3.1)   ALU SOURCE SELECTOR :

___________________________________________________

|      BIT        |                               |

|     FIELD       |  ALU SOURCE OPERANDS SELECTED |

|                 |-------------------------------|

|   I2 I1 I0      |       RE       |      S       |

|-----------------|----------------|--------------|

|      000        |       A        |      Q       |

|      001        |       A        |      B       |

|      010        |       0        |      Q       |

|      011        |       0        |      B       |

|      100        |       0        |      A       |

|      101        |       D        |      A       |

|      110        |       D        |      Q       |

|      111        |       D        |      0       |

|_________________|________________|______________|

NOTE : RE and S are the two outputs of the ALU source selector.

(1.3.2) ALU FUNCTION :

_____________________________________________________

|      BIT        |                                 |

|     FIELD       |  ALU FUNCTION ( OUTPUT --> F)   |

|                 |---------------------------------|

|   I5 I4 I3      |     C0 = 0      |    C0 = 1     |

|-----------------|-----------------|---------------|

|      000        |   RE + S        | RE + S + 1    |

|      001        |   S - RE - 1    | S - RE        |

|      010        |   RE - S - 1    | RE - S        |

|      011        |   RE or S       | RE or S       |

|      100        |   RE and S      | RE and S      |

|      101        |   not(RE) and S | not(RE) and S |

|      110        |   RE xor S      | RE xor S      |

|      111        |   RE xnor S     | RE xnor S     |

|_________________|_________________|_______________|

NOTE : C0 is the carry-in input.

F is the output of the ALU.

(1.3.3) ALU DESTINATION :

__________________________________________________________________________

|    BIT   |RAM (Baddress)|    Q-REG.    |        |    RAM     |  Q-REG. |

|   FIELD  |   FUNCTION   |   FUNCTION   |   Y    |  SHIFTER   | SHIFTER |

|          |--------------|--------------|        |------------|---------|

| I8 I7 I6 | SHIFT | LOAD | SHIFT | LOAD |        |RAM0 | RAM3 | Q0 | Q3 |

|----------|-------|------|-------|------|--------|-----|------|----|----|

|   000    |  ---  | ---  | ---   |   F  |    F   | --- | ---  |--- |--- |

|   001    |  ---  | ---  |  ---  | ---  |    F   | --- | ---  |--- |--- |

|   010    | ---   |  F   |  ---  | ---  |    A   | --- | ---  |--- |--- |

|   011    | ---   |  F   |  ---  | ---  |    F   | --- | ---  |--- |--- |

|   100    | down  |  F/2 | down  |  Q/2 |    F   | out |  in  |out | in |

|   101    | down  |  F/2 |  ---  | ---  |    F   | out |  in  |out |--- |

|   110    |  up   |  2F  |  up   |  2Q  |    F   |  in | out  | in |out |

|   111    |  up   |  2F  |  ---  | ---  |    F   |  in | out  |--- |out |

|__________|_______|______|_______|______|________|_____|______|____|____|

NOTE :  Data that is loaded into the RAM is written at the RAM word pointed

to by the address input Badd.

Note that the bidirectional ports are active only when some

shifting is being done.

Whenever a bidirectional port is NOT being used as an output by

the Am2901, the Am2901 chip tristates it from its own side. Then,

it can be used as an input from the external world or it may be

left inactive. To deal with this phenomenon in our models, we

have a resolution function that resolves the values written by the

two drivers ( the Am2901 chip and the external world).

===========================================================================

2)  MODELS DEVELOPED FOR THE Am2901

process ---> This sub-directory contains the file in which the Am2901 is

modeled as a single VHDL process.

block   ---> This sub-directory contains the file inwhich the Am2901 is

partitioned into functional blocks and the blocks are modeled as

VHDL concurrent blocks.

===========================================================================

3) TESTING STRATEGY

Each of the above models are tested with the set of test vectors in

file "test_vectors.vhdl".

The test vectors were chosen specifically to verify the functionality

of different portions of the chip in different modes of operation and

data transfer along different paths in the model.

Each of the three bit fields in the instruction is tested at least once in

each of its eight modes in these test vectors.

Those models that have constituent components (e.g. models in "block/")

have test vector sets for each of those components.

Please see the "ROADMAP" file in the parent of the present directory

( "2901/") to get an idea where those files are located.

===========================================================================

4) STATUS OF MODELS

__________________________________________________________________________

|                  |                   |           |        |            |

| MODEL            | TEST VECTOR USED  | SIMULATOR | ERRORS | SYNTHESIS  |

|__________________|___________________|___________|________|____________|

|                  |                   |           |        |            |

|  block           | test_vectors.vhdl | Zycad 1.0 | None   | ----       |

|__________________|___________________|___________|________|____________|

|                  |                   |           |        |            |

|  process         | test_vectors.vhdl | Zycad 1.0 | None   | ----       |

|__________________|___________________|___________|________|____________|